MICROPLUS – SISTEMA CONCELEAD 400

Voce di Capitolato:

I pannelli MICROPLUS di tipologia SUPREME con fori diametro 0,5mm in parallelo con passo 2 mm, 250.000 fori per metri quadrato e di tipologia MEDIUM con fori diametro 0,5mm in parallelo con passo 6 mm, 28.000 fori per metro quadrato, installati con sistema CONCELEAD 400 sono con bordo con fresatura maschio/femmina per struttura nascosta. Realizzati in MDF ignifugo nobilitato melaminico (B s2 d0) o impiallacciato in legno (B s2 d0/ B s1 d0), spessore 14mm. Tutto il sistema di controsoffitto dispone delle seguenti certificazioni: FSC (materiale riciclato al 100%), EPD (dichiarazione ambientale di prodotto), ISO 9001, Marcatura CE normativa 13964: 2014, protocollo LEED. I componenti forniti saranno corredati da D.O.P. Dichiarazione di Prestazione CE in conformità al regolamento Europeo dei prodotti da costruzione 305/2011. La finitura dei pannelli sarà scelta dalla Direzione Lavori in base alla cartella delle finiture CBI EUROPE. Finiture melaminiche: Rovere Rock Bianco - Olmo Tafira - Rovere Italia - Rovere Bruges - Noce Athens - Faggio Bavaria - Bianco Standard. Finitura impiallacciata colore a scelta della Direzione Lavori. Le dimensioni dei pannelli saranno standard 256x1792mm oppure a richiesta come da indicazioni delle schede tecniche CBI EUROPE. Ogni pannello sarà fornito con applicazione di tessuto fonoassorbente ignifugo applicato sul retro del peso di 65 gr/mq di colore nero. Per aumentare le prestazioni acustiche nell’intradosso del controsoffitto potrà essere inserito un materassino in lana di roccia spessore 40 mm e densità 40 Kg/m³. Ogni pannello sarà installato in appoggio su specifica struttura di sostegno SISTEMA CONCELEAD 400 prodotta da CBI EUROPE composta da: profili portanti in lamiera zincata tipo P3700Z sezione a “T” dimensioni 24x38mm. I pannelli sono agganciati al profilo mediante il gancio di sospensione tipo SP76. Sospensione al soffitto esistente mediante pendino rigido d’acciaio zincato 4 mm e molla d’acciaio armonico. Lungo il perimetro il controsoffitto verrà rifinito con apposito profilo angolare in acciaio o alluminio preverniciato.

Specification:

MICROPLUS panels of the SUPREME type with 0.5mm diameter holes in parallel with a 2mm pitch, 250000 holes per square meter and of the MEDIUM type with 0.5mm diameter holes in parallel with 6mm pitch, 28000 holes per square metre, installed with CONCELEAD 400 are with milled pins and tails edge for hidden structure. Made of fire retardant melamine faced MDF (B s2 d0) or wood veneer (B s2 d0/B s1 d0), thickness 14 mm. The entire false ceiling system has the following certification: FSC

(100% recycled material), EPD (enviromental product declaretion), ISO 9001, CE marketing, standard 13964: 2014, LEED protocol. The supplied components will be accompanied by D.O.P. CE Declaration of Performance in accordance with the European regulation of construction products 305/2011. The finish of the panels will be chosen by the Works Management based on te CBI EUROPE finish folder. Melamine finishes: White Rock Oak – Tarifa Elm – Italian Oak – Bruges Oak – Athens Walnut – Bavarian Beech – Standard White. Color veneer finish chosen by the Works Management. The dimensions of the panels will be 600x600mm, 600x1200mm, 256x1792mm or on request as indicated in the CBI EUROPE technical data sheets. Each panel can be completely inspected and supplied with a smooth, perforated or slotted surface (see CBI EUROPE perforations and slots). Each panel will be supplied with application of fireproof sound-absorbing fabric applied on the back weighing 65 gr/mq in black colour. To increase the acoustic performance, a 40mm thick rock wool mat with a density of 40 Kg/m³ can be inserted into the intrados of the false ceiling. Each panel will be installed resting on a specific support structure CONCELEAD 400 produced by CBI EUROPE composed of: bearing profiles in galvanized steel type P 3700 Z “T” section dimensions 24x38mm. The panels are hooked to the profile using the SP 76 type suspension hook. Sunspension to the existing ceiling by means of a rigid 4 mm galvanized steel hanger and harmonic steel spring. Along the perimeter, the false ceiling will be finished with a special angular profile in pre-painted steel or aluminum.