MICROPLUS – COVERING 500

Voce di Capitolato:

I pannelli MICROPLUS di tipologia SUPREME con fori diametro 0,5mm in parallelo con passo 2 mm, 250.000 fori per metro quadrato e di tipologia MEDIUM con fori diametro 0,5mm in parallelo con passo 6 mm, 28.000 fori per metro quadrato, installati con sistema COVERING 500 sono con bordo con fresatura maschio/femmina per struttura nascosta. Realizzati in MDF ignifugo nobilitato melaminico (B s2 d0) o impiallacciato in legno (B s2 d0/ B s1 d0), spessore 14mm. Tutto il sistema di controsoffitto dispone delle seguenti certificazioni: FSC (materiale riciclato al 100%), EPD (dichiarazione ambientale di prodotto), ISO 9001, Marcatura CE normativa 13964: 2014, protocollo LEED. I componenti forniti saranno corredati da D.O.P. Dichiarazione di Prestazione CE in conformità al regolamento Europeo dei prodotti da costruzione 305/2011. La finitura dei pannelli sarà scelta dalla Direzione Lavori in base alla cartella delle finiture CBI EUROPE. Finiture melaminiche: Rovere Rock Bianco - Olmo Tafira - Rovere Italia - Rovere Bruges - Noce Athens - Faggio Bavaria - Bianco Standard. Finitura impiallacciata colore a scelta della Direzione Lavori. Le dimensioni dei pannelli saranno 256x1792mm o a richiesta come da indicazioni delle schede tecniche CBI EUROPE. Ogni pannello sarà completamente ispezionabile e fornito a superficie liscia, forata o asolata (vedi forature e asolature CBI EUROPE). Ogni pannello sarà fornito con applicazione di tessuto fonoassorbente ignifugo applicato sul retro del peso di 65 gr/mq di colore nero. Per aumentare le prestazioni acustiche nell’intradosso del controsoffitto potrà essere inserito un materassino in lana di roccia spessore 40 mm e densità 40 Kg/m³. Ogni pannello sarà installato in appoggio su specifica struttura di sostegno SISTEMA COVERING 500 prodotta da CBI EUROPE composta da: profili primario tipo M46/75 in lamiera zincata sezione ad “U” dimensioni 46x75x44mm e profili in alluminio estruso PP1001/PP1001.N sezione ad “H” dimensioni 36,5x9x38mm. Come finitura base e altezza della parete si usano i profili in alluminio estruso PP1002/PP1003 di sezione a “U” dimensioni 28x20,2x13mm.

Specification:

MICROPLUS panels of the SUPREME type with 0.5mm diameter holes in parallel with a 2mm pitch, 250000 holes per square meter and of the MEDIUM type with 0.5mm diameter holes in parallel with 6mm pitch, 28000 holes per square metre, installed with COVERING 500 are with edge with pins and tails milling for hidden structure. Made of fire retardant melamine faced MDF (B s2 d0) or wood veneer (B s2 d0/B s1 d0), thickness 14 mm. The entire false ceiling system has the following certification: FSC (100% recycled material), EPD (enviromental product declaretion), ISO 9001, CE marketing, standard 13964: 2014, LEED protocol. The supplied components will be accompanied by D.O.P. CE Declaration of Performance in accordance with the European regulation of construction products 305/2011. The finish of the panels will be chosen by the Works Management based on te CBI EUROPE finish folder. Melamine finishes: White Rock Oak – Tarifa Elm– Italian Oak – Bruges Oak – Athens Walnut – Bavarian Beech – Standard White. Color veneer finish chosen by the Works Management. The dimensions of the panels will be 256x1792mm or on request as indicated in the CBI EUROPE technical data sheets. Each panel can be completely inspected and supplied with a smooth, perforated or slotted surface (see CBI EUROPE perforations and slots). Each panel will be supplied with application of fireproof sound-absorbing fabric applied on the back weighing 65 gr/mq in black colour. To increase the acoustic performance, a 40mm thick rock wool mat with a density of 40 Kg/m³ can be inserted into the intrados of the false ceiling. Each panel will be installed resting on a specific support structure COVERING 500 SYSTEM produced by CBI EUROPE composed of: primary profiles in galvanized steel type M46/75 section “U” dimensions 46x75x44mm and profiles in extruded aluminum type PP 1001/PP 1001.N section “H” dimensions 36.5x9x38mm. The PP 1002/PP 1003 extruded aluminum profiles with a “U” section, dimensions 28x20.2x13mm, are used as the basic finish and height of the wall.