IRON GENESY

voci di capitolato

Il sistema Iron Genesy è costituito da un telaio strutturale, composto da montanti verticali 70 x 56 mm, in acciaio zincato di 1,0 mm di spessore, collegato meccanicamente, e dotato di supporti regolabili per consentire il livellamento su superfici irregolari. I montanti sono forniti con perforazioni ad intervalli regolari per il passaggio di cavi elettrici e cavi per apparecchiature di teleco­municazione. I moduli standard hanno spessore di 101 mm e una larghezza di 1215 mm. L’altezza può essere variabile.

Le lastre di vetro per i telai vetrati possono essere sp. 5 mm vetro float o vetro stratificato di sicurezza 6/7 mm composto da due lastre da 3 mm con interposto un strato di PVB sp. 0.38 mm. Il vetro di sicurezza stratificato (UNI EN ISO 12543-UNI EN 12600) fornisce diversi livelli di sicurezza grazie al plastico in polivinilbutirrale (PVB), che impediscono alle schegge di staccarsi in caso della rottura del vetro (protezione di sicurezza) o impedire ad un corpo di attraversare il vetro (protezione contro la caduta). Le ante vetrate delle porte sono prodotte usando un vetro da 10 mm temperato a filo lucido.

specifications

Iron Genesy Demountable Partition system consists of a structural frame, composed by vertical posts 70 x 56 mm, made in galvanized steel’ of 1,0 mm thickness, mechanically connected, and provided with adjustable supports to allow for easy leveling on uneven surfaces.

The posts are provided with perforations at regular intervals for the passage of all electrical cables and wires for telecommunication equipment. The standard partition modules have a thickness of 101 mm and a width of 1215 mm. The height can be variable.

The glass sheets for the windows could be 5 mm tempered float glass sheet, or a 6/7 mm laminated safety glass composed by n° 2 sheets of 3 mm glass and transparent PVB 0.38 mm thick. plastic.

The laminated safety glass (UNI EN ISO 12543-UNI EN 12600) provide different levels of protection thanks to plastic interlayers in polyvinyl butyral (PVB) whose purpose is to prevent splinters from detaching (safety protection) or that a human body can go through glass (protection against falling). Glazed leaf for glass doors are manufactured with 10 mm tempered glass.